Giving your child a Hydrocortisone injections using the Solu-Cortef Acto-vial

You have been taught to give a Hydrocortisone (Solu-Cortef) injection.  You would give this to your child if they are vomiting, have diarrhea, are unconscious or have a serious injury such as a broken bone.

You will need these supplies from your Emergency injection bag

O  23 gauge 1” IM needle     or    O   25 gauge 5/8 “needle

O     5ml syringe                        or    O    3ml syringe

2 Chlorhexidine or alcohol wipes, 1 sterile gauze, and 1 Band-Aid

- Solu-Cortef   Act-o-vial  100mg /2ml preparation Yellow cap DIN 00030600

O  Solu-Cortef   Act-o-vial  250mg /2ml preparation  Red cap DIN 00030619

Amount to be given:    ________________________________________________

To prepare the injection

- Thoroughly clean the area where you will be preparing your injection

- Wash your hands

- Firmly press down on the top of the Act-o-vial to force the diluent(clear liquid)from the top compartment  into the lower compartment onto the powder

- Gently rotate or swirl the Act-o-vial to mix the liquid and the powder and ensure there are no white unmixed lumps. Then let solution sit for a minute until it becomes clear.

- Remove the small plastic tab covering the centre of the stopper and cleanse with one of the wipes.

- As you were shown , attach the needle to the syringe ensuring that both pieces remain sterile where they connect to each other

- Remove the cap of the needle by grasping the cap at the end and carefully pulling it off and place it in the clean area

- Insert or push the needle through the centre of the stopper.

- Invert the vial ( turn upside down) and withdraw the dose of medication slowly

- Push the air out of the syringe to ensure your dose is only the medication. This may take a few tries to complete

- Place cap back on needle to protect it from contamination

To give the injection to your child

- Position your child so that the largest area of the thigh is exposed

- Clean the skin in a circular motion with one of the wipes as you were shown

- Remove the cap from the needle by grasping the end and pulling it off carefully

- Hold the injection in your dominant hand, support the skin on the leg with the other hand

- Insert the needle at a 90-degre angle with a quick dart-like motion

- Inject medication at an even rate, approximately under 30 seconds

- Withdraw needle rapidly and using sterile gauze apply pressure to the injection site, bleeding is normal

- Apply Band-Aid if you prefer

- As instructed , using push tab on the needle to cover and close the needle end

- Place needle in sharps container or solid plastic container for disposal

For emergencies please contact the Endocrinologist on call at 613-737-7600-0

If you have questions during business hours please contact your Endocrine Nurse

__________________ 613-737-7600 x ______